Cyber Netics Club

Packet Tracer Lab: Build Your First Network

Beginner Level

Name: ______________________________     Date: ________________

# Goal

Build a small network from scratch: connect two LANs with a router, add a web server, and prove it all works. Along the way you will cable devices, assign IP addresses, configure a router and a switch, watch packets travel, and lock down your devices with basic security.

# Key Terms

| Term | What it means |
| --- | --- |
| LAN | Local Area Network: devices in one small area (a room, a school lab, a home). |
| IP address | A logical address that identifies a device on a network (like 192.168.10.10). |
| Subnet mask | Tells a device which part of an IP address is the network and which part is the host. |
| Default gateway | The router address a device uses to reach other networks. |
| MAC address | A hardware address burned into a network card. Switches use these. |
| Switch | Connects devices inside a LAN and forwards frames using MAC addresses. |
| Router | Connects different networks and forwards packets using IP addresses. |
| Server | A computer that provides a service (like web pages) to clients. |
| Ping | A test that sends a small message to see if another device replies. |
| ARP | A process that finds the MAC address that goes with an IP address. |

# Cable Reference

| Cable | Used for | Notes |
| --- | --- | --- |
| Straight-through | Connecting different device types (PC to switch, switch to router) | Copper, up to about 100 m |
| Crossover | Connecting like devices (switch to switch, PC to PC) | Copper, up to about 100 m |
| Console (rollover) | Managing a switch or router from a PC's console port | Used for the first setup |
| Fiber optic | Long distances and high speeds | Uses light instead of electricity, bigger capacity |

Questions

Q1. Which cable would you normally use to connect two switches together?

Q2. Name one advantage fiber has over copper.

# Topology

PC1 ---+                                +--- PC3

+--- S1 ---- R1 (2 ports) ---- S2 +

PC2 ---+                                +--- Server

LAN A: 192.168.10.0/24         LAN B: 192.168.20.0/24

Devices: 1 router (2911), 2 switches (2960), 3 PCs, 1 server. All links use Copper Straight-Through cable.

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 | n/a |
| R1 | G0/1 | 192.168.20.1 | 255.255.255.0 | n/a |
| PC1 | NIC | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC2 | NIC | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| PC3 | NIC | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| Server | NIC | 192.168.20.100 | 255.255.255.0 | 192.168.20.1 |

Tip: Save your .pkt file after each part (File > Save).

# Part 0: Place Devices (5 min)

- Open Packet Tracer and start a new file.

- From the bottom-left device panel, drag out 1 router (2911), 2 switches (2960), 3 PCs, and 1 server.

- Arrange them like the topology diagram.

- Rename each device by clicking its label: R1, S1, S2, PC1, PC2, PC3, Server.

# Part 1: Cabling (8 min)

Choose the Copper Straight-Through cable (solid black line) from the Connections panel (lightning bolt icon). Connect:

| From | To | Ports |
| --- | --- | --- |
| PC1 | S1 | FastEthernet0 to Fa0/1 |
| PC2 | S1 | FastEthernet0 to Fa0/2 |
| S1 | R1 | Fa0/24 to G0/0 |
| PC3 | S2 | FastEthernet0 to Fa0/1 |
| Server | S2 | FastEthernet0 to Fa0/2 |
| S2 | R1 | Fa0/24 to G0/1 |

Note: Link lights start red or amber and turn green after about 30 seconds. The router links stay red until Part 3.

Questions

Q3. Why do we use straight-through cables here instead of crossover?

Q4. Which device connects devices within a LAN: the switch or the router?

Q5. Which device connects different networks?

Q6. What does a red or amber link light tell you compared with a green one?

# Part 2: Configure PCs and Server (8 min)

- Click a PC, then open the Desktop tab, then IP Configuration.

- Select Static and enter the IP address, subnet mask, and default gateway from the addressing table.

- Repeat for PC2, PC3, and the Server (the Server also uses Desktop > IP Configuration).

Questions

Q7. What does the default gateway do?

Q8. Why are PC1 and PC3 on different networks? (Hint: look at the third number.)

Q9. With the mask 255.255.255.0, which part of 192.168.10.10 is the network and which part is the host?

# Part 3: Configure the Router (12 min)

Click R1, then the CLI tab. If asked to continue with the initial configuration dialog, type no and press Enter. Then type these commands:

enable

configure terminal

hostname R1

interface g0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

end

copy running-config startup-config

Verify with show ip interface brief. Both interfaces should say up / up. Then run show ip route and look at the routes listed.

Questions

Q10. What does no shutdown do?

Q11. Router interfaces are off by default. Why might that be a good security default?

Q12. In show ip route, what does the letter C at the start of a line mean?

Q13. Why do we run copy running-config startup-config?

# Part 4: Test and Troubleshoot (8 min)

On PC1, open Desktop > Command Prompt and run:

ping 192.168.10.11     (PC2, same LAN)

ping 192.168.20.10     (PC3, other LAN)

ping 192.168.20.100    (Server)

Note: The first ping in a series often times out. That is normal; the network is still learning addresses (ARP).

Challenge: On PC1, clear the default gateway (IP Configuration) and ping PC3 again. Then put the gateway back.

Questions

Q14. With no gateway, did pinging PC2 still work? Did pinging PC3?

Q15. Why the difference?

Q16. Compare the TTL in a reply from PC2 with a reply from PC3. What is different, and what does it tell you?

# Part 5: Web Server (7 min)

- Click the Server, then Services > HTTP. Make sure HTTP is On.

- Click Edit next to index.html and change the text to include your name. Save it.

- On PC1, open Desktop > Web Browser and go to http://192.168.20.100.

Questions

Q17. Which device serves the page and which requests it (client or server)?

Q18. Did the traffic cross the router? How do you know?

# Part 6: Simulation Mode (8 min)

- Click the Simulation tab (bottom right). Click Edit Filters and show only ICMP and ARP.

- Ping the server from PC1. Click Capture/Forward one step at a time.

- Click on the packet envelopes to inspect them at each device.

Questions

Q19. What was the first type of packet sent: ARP or ICMP?

Q20. List the devices the packet passed through between PC1 and the Server.

Q21. Which OSI layer does the switch work at? The router?

# Part 7: Basic Security (5 min)

On R1's CLI:

enable

configure terminal

enable password cisco123

end

show running-config

Questions

Q22. Can you read the password in the config?

Now run configure terminal, then enable secret cisco456, then no enable password, then end, and look at show running-config again.

Q23. What changed?

# Part 8: Wrap-Up (5 min)

Questions

Q24. What is the difference between a switch, a router, and a server?

Q25. What was one thing that went wrong or confused you, and how did you fix it?

Q26. Name one real place where you would find a network like this.